Commands

NOTE: All commands are case insensitive.

- wait n

  - Wait n milliseconds.

  - UART output: The command will print “Delaying n milliseconds…” and will print “ Finished” upon completion.

  - Example:

    - wait 250

    - Output: Delaying 250 milliseconds… Finished

- gpio port [value]

  - Read or write to the specified port.

  - Without value the specified port is read and the value is printed in binary (with '_' every 4 bits for clarity).

  - With value the specified value is written to the port.

  - port can be A, B, C, D, E, F, or G for the MCU onboard GPIO ports or XA, XB, XC, XD for the GPIO expanders.

  - Onboard ports are 8 bits while expander ports are 16 bit.

  - value with be 8 or 16 bit accordingly.

  - Each digit can be one of the following:

    - 0: output a 0 to this bit

    - 1: output a 1 to this bit

    - z: configure this bit for input

    - x: leave this bit alone

  - All underscores in the input value are ignored.

  - UART output:

    - Write: The command will print “Writing to *port*…” where port is either GPIO expander X or port X. The command will then print the values of the input mask, the high output mask, and the low output mask in hexadecimal format (with leading 0x). Upon completion the command will print “Finished setting *port*”

      - Example: gpio xc xxxx_zzzz_11xx_0000

        - Writing to GPIO expander C…

        - Inputs: 0x0F00

        - High Outputs: 0x00C0

        - Low Outputs: 0x000F

        - Finished setting GPIO expander C

    - Read: The command will print “Reading from *port*…” where port is either GPIO expander X or port X. The command will then print the input values and output values in binary format.

      - Example: gpio a

        - Reading from port A…

        - Inputs: XXXX_XXXX

        - Outputs: XXXX_1XXX

  - Examples:

    - gpio a 1xx0_zz00

    - gpio B

    - gpio XA 1100_0011_xxxx_zzzz

    - gpio xc

- spi port. cs value

  - Simultaneously read and write value to the specified SPI port.

  - port can be 0 or 1 to indicate which SPI interface to use.

  - cs indicates to chip select that will be used for SPI communication.

  - value should be given in bytes (in hexadecimal format) separated by spaces.

  - UART output: The command will print “Sending data through SPI port CS cs…”. It will then print “Sending value…” before sending each value and “ Sent” upon a successful send of the value. It will then print “Reading results…” followed by printing all the results on one line, in hexadecimal format, separated by spaces.

    - Example: spi 0.5 41 12 00 00 00 00

      - Sending data through SPI0 CS5…

      - Sending 0x41… Sent

      - Sending 0x 12… Sent

      - Sending 0x 00… Sent

      - Sending 0x 00… Sent

      - Sending 0x 00… Sent

      - Sending 0x 00… Sent

      - Reading results…

      - Results: 41 12 80 00 80 00 7F

  - Examples:

    - spi 0.3 41 00 01 02 03

    - spi 1.0 00 00 01 00

- i2c port. address ( value or r)

  - Performs read and write operations on the specified I2C port.

  - If value is given, the specified value is written to the specified device.

  - If r is given in place of value, a read operation is performed on the specified device.

  - port can be 0 or 1 to indicate which I2C interface to use. Realisitically this will always be 1 as I2C0 will operate in slave mode.

  - Address is the slave address of the specified device (in hexadecimal format).

  - UART output:

    - Write: The command will print “Writing to I2C port Slave: address…”. The command will then print “Sending value…” before sending each value. Upon a successful send, the command will print “ Sent.” If the I2C times out, however, the command will print out an error, and exit. After a complete successful send, the command will print “Send command complete.”

      - Example: i2c 1.0 00 00 00

        - Writing to I2C1 Slave: 0x00…

        - Sending 00… Sent

        - Sending 00… Error: I2C Send timed out

        - Exiting I2C command

    - Read: The command will print “Reading from I2C port Slave: address...” and will then begin printing the data received.

      - Example: i2c 1.fa

        - Reading from I2C1 Slave: 0xFA…

        - Data: 6E

  - Examples:

    - i2c 1.F0 r

    - i2c 1.52 41 F3 65

- clkreg clk. reg [ _data_]

  - Reads or writes to the specified clock register.

  - data is a 16 bit value, and can be listed as octal, decimal, or hexadecimal. Using a leading 0 will indicate octal, while using a leading 0x will indicate hex.

  - If data is given, the specified data is written to the specified clock register.

  - If data is not given, the specified clock register is read and the result output on the terminal.

  - clk specifies the clock to be used. Clocks range from 1 - 3.

  - reg specifies the register to be operated on. The register can be listed as octal, decimal, or hexadecimal. Using a leading 0 will indicate octal, while using a leading 0x will indicate hex.

  - UART output:

    - Write: The command will print out “Writing value to register reg on clock module clk…” and append “ Finished” to this line when the write is successful.

      - Example: clk 1.0 0x020F

        - Writing 0x020F to register 0 on clock module 1… Finished

    - Read: The command will print out “Reading from register reg on clock module clk…” and then output the data on the following line.

      - Example: clk 3.5

        - Reading from register 5 on clock module 3…

        - Data: FE56

  - Examples

    - clkreg 1.0 0x020F

    - clkreg 3.5 017

    - clkreg 2.4

- lcd string page line

  - The LCD works through a page system. There are a total of 16 pages (starting at page 0 and ending at page 15), each containing two lines (0 and 1). The command works by writing the given string to the specified page and line.

  - Note: Currently all strings printed through this command are printed in lower case).

  - UART output: Prints the line “Adding “string” to page page line line…” and adds “ Finished” upon completion

  - Example:

    - lcd hello 7 0

    - Output: “Adding “hello” to page 7 line 0… Finished”

- setboot mode

  - Sets the bootmode to the value of mode.

  - Mode is a 32 bit value represented in hexadecimal. See this page for information on various bit values.

  - Example:

    - Setboot 10AF1EE1

- warmrst

- fullrst

  - Places the board in reset for 1 second, places the bootmode on the GPIO pins, and then takes the board out of reset.

  - Example:

    - Fullrst

- Eeprom addr [bytes…]

  - Writes and reads to the EEPROM

  - If [bytes..] are given as byte-wide values in hexadecimal separated by spaces, then the values are programmed into EEPROM starting at addr

  - If [bytes…] are not given, then the first 16 bytes starting at address are read from eeprom and output on a single line separated by spaces

  - Addr is given as a 2-byte-wide hex value.

  - Example:

    - Eeprom 0F56 A5 F8 70

    - Eeprom 0065

- Commission board_type board_version board_sn

  - Stores the 3 values (given in hexadecimal) into the OTP flash registers, and sets up the BOOTCFG register to allow in-field firmware updates via UART. If these registers have already been programmed (the command has been used before), this command is disabled. Once the command is used, the MCU must be hard reset for the command to take effect.

  - To reset these registers see page 175 of the Stellaris Data Sheet. This process can also be done easily through the LM Flash Programmer in the Other Utilities tab.

  - To perform an in-field update, connect the MCU_BOOTSELECT jumper to ground and reset the MCU. Make sure you are connected to the MCU UART, and use the LM Flash Programmer (available here) to update the firmware, as detailed below.

    - In the ‘Configuration’ tab, in the interface section, select ‘Serial (UART)’ from the drop-down box on the left.

    - Select the appropriate COM Port, and set the ‘Baud Rate’ to 115200.

    - Set ‘Transfer Size’ to 60, and make sure ‘Disable Auto Baud Support’ is unchecked.

    - In the ‘Program’ tab, Browse to the location of the binary file containing the firmware update, and select it.

    - Leave all other options as default, and press the ‘Program’ button.

    - After the programmer is complete, disconnect the jumper from ground and reset the board.

- Error enable

  - Enables and disables error handling in EVM setup.

  - If enable is 1 then error handling is enabled; if it is 0 then error handling is disabled.

  - If error handling is enabled during setup the BMC will stop setup and exit if anytime an error occurs, without completing setup. If error handling is disabled the BMC will output a message to the UART console and continue performing setup.

  - By default error handling is enabled.

  - This command is volatile and resets to enabled on reset. To perform setup without error handling enabled the ‘pwr’ command will need to be used.

- Pwr

  - This command performs EVM setup.

- Ver

  - Displays the BMC version, board type, board version, and board serial number in a formatted way on the UART console.